MSPM’s

Deogiri Institute of Engineering and Management Studies,

Aurangabad

Department of Computer Science and Engineering

Survey Based Project Report on

Dell Inspiron3800 and Dell Inspiron  15 5555

Subject: Computer Architecture and Organization

Submitted by

Name of Student:1.Swapna Subhash Salunke, Roll No. 26159

2. Priyanka Dattatray Tayade, Roll No. 26160

Under the Guidance of

Prof. P.H. Durole

Asst. Prof.

Department of Computer Science and Engineering

(Deogiri Institute of Engineering and Management Studies)

CERTIFICATE

This is to certify that Miss. Swapna Subhash Salunke (Roll No. 26159) and Priyanka Dattatray Tayade had successfully completed her Survey Based Project on Dell Inspiron 3800  and Dell Inspiron  15 5555on due 28-09-2019 (date of the submission).

Index

| Sr. No. | Chapter | Page No. |
| --- | --- | --- |
| 1. | Introduction To Dell Inspiron 3800 | 4 |
| 2. | View | 5 |
| 3. | Introduction To Dell Inspiron 15 5000 | 7 |
| 4. | View | 8 |
| 5. | Processor Architecture | 13 |
| 6. | Register Set | 14 |
| 7. | Instruction Set | 18 |
| 8. | Memory Organization | 21 |
| 9. | Modes of Operation | 23 |
| 10. | IO Mechanism | 27 |
| 11. | Compare | 31 |
| 12. | Quotation | 32 |

Introduction

Dell Inspiron 3800

Released in 2000, the Dell Inspiron 3800 was an affordable laptop that Dell branded as "Stylish and Affordable." Its starting price was $1,199.

- Processor: Intel Pentium III @700 MHz, Intel Pentium III @600 MHz, Intel Celeron @600 MHz, or Intel Celeron @700 MHz

- Memory: 32, 64, 96, 128, 192, 256, 384 or 512 MB of DDR RAM

- Graphics: ATI 2x AGP Rage Mobility with 8mb of video memory

- Display: 12.1" 800x600, or a 14.1" 1024x768

- Storage: 5gb, 6gb, 10gb, or 20gb Ultra ATA hard drive

View

Dell Inspiron 3800

Front View of the Computer

- Display latch                                                     9. DualPoint Buttons

- Air intake 10.Battery Bay

- Modem connector		11.Touch Pad

- TV-out S-video connector		12.Keyboard

- Audio connectors (3)				13.Track Stick

- Integrated microphone		14.Power Button

- Speaker					15.Keyboard status indicator

- Media bay					16. Display

Back View of the Computer

1. Speaker						9.  PS/2 connector

2. Security cable slot					10. USB connector

3. Hard-disk drive					11. Docking connector

4. PC Card slot 					12. Device access/status indicators

5. Security cable slot	                                                 13. Parallel port connector

6. Infrared port					 14. Video connector

7. AC adapter connector			15. Fan exhaust

8. Serial port connector

Introduction

Dell Inspiron 5555

Dell Inspiron 5555 is a Windows 10 Home laptop with a 15.60-inch display that has a resolution of 1366x768 pixels. It is powered by a APU Quad Core A10 processor and it comes with 8GB of RAM. Graphics are powered by AMD Radeon R6 M345DX.

- Processor: AMD E2-7110 APU, AMD A4-7210 APU @1.8 GHz, AMD A6-7310 APU @2 GHz, AMD A8-7410 APU @2.2 GHz, and AMD A10-8700P APU @1.8 GHz

- Memory: 4 GB, 8 GB, 12 GB, or 16 GB DDR3L 1600 MHz RAM

- Graphics Processors: AMD Radeon R2, R3, R4, R5, or R6 Integrated Graphics

- System: x64 based-processor

- Display: Non-touch or touch 15.6" Dell TrueLife 1366x768 display

- Storage: Seagate Laptop Thin HDD SATA 500 GB, 1 TB, or 2 TB (5400 RPM) or 128 GB and 256 GB SSD

- Optical Drive: TSSCorp DVD-RW

- Battery: 4-cell 40WHr or 47WHr Lithium-Ion Battery

- Camera: HD 720p Web Camera with stereo microphones

- Wi-Fi and Bluetooth: Dell Wireless 1707 Card (802.1b/g/n + Bluetooth 4.0, 2.4 GHz)

- I/O ports: 1 USB 3.0, 2 USB 2.0, 1 HDMI 1.4a port, SD Card Reader, 1 Ethernet port, 1 combined headphone and microphone jack, 1 security lock, 1 power adapter connection port.

- Keyboard: Full-size non-backlit or backlit spill-resistant keyboard w/ number pad

View

Dell Inspiron 5555

Front View

- Power and battery-status light/hard-drive activity light

Indicates  the  power  and  battery-charge status. Press Fn+H to toggle this light between hard-drive activity light and battery-status light.

Indicates the power and battery-charge status. Press Fn+H to toggle this light between hard-drive activity light and battery-status light.

Solid White                Power adapter is connected and battery has more than 5%.

Amber          Computer is running on battery and battery has less than 5%.

Off                            Power adapter is connected and the battery is full charged

Left  View

- Power-adapter port

Connect a power adapter to provide power to your computer and charge the battery.

- Network port

Connect an Ethernet (RJ45) cable from a router or a broadband modem for network or internet access.

- HDMI port

Connect a TV or another HDMI-in enabled device. Provides video and audio output.

- USB 3.0 port

Connect peripherals such as storage devices, printers, and so on. Provides data transfer speeds up to 5 Gbps.

- Media-card reader

Reads from and writes to media cards.

Right View

- Headset port

Connect a headphone, a microphone, or a headphone and microphone combo (headset).

- USB 2.0 ports (2)

Connect peripherals such as storage devices, printers, and so on. Provides data transfer speeds up to 480 Mbps.

- Optical drive

Reads from and writes to CDs and DVDs.

- Security-cable slot

Connect a security cable to prevent unauthorized movement of your computer.

Base view

- Left-click area

Press to left click.

- Touch pad

Move your finger on the touch pad to move the mouse pointer. Tap to left click.

- Right-click area

Press to right click

- Power button

Press to turn on the computer if it is turned off, in sleep state, or in hibernate state.

Press to put the computer in Sleep state if it is turned on.

Press and hold for 4 seconds to force shut-down the computer..

Processor Architecture

Dell inspiron 3800

The Pentium Pro processor introduced a new micro-architecture for the Intel IA-3 processors,commonly referred to as P6 processor micro architecture. The P6 processor microarchitecturewas later enhanced with an on-die, 2nd level cache, called Advanced Transfer Cache. Thismicro-architecture is a three-way superscalar, pipelined architecture. The term “three-waysuperscalar” means that using parallel processing techniques, the processor is able on averageto decode, dispatch, and complete execution of (retire) three instructions per clock cycle. Tohandle this level of instruction throughput, the P6 processor family use a decoupled, 12-stagesuper pipeline that supports out-of-order instruction execution. Figure 2-1 shows a conceptualview of the P6 processor micro-architecture pipeline with the Advanced Transfer Cacheenhancement. The micro-architecture pipeline is divided into four sections (the 1st level and 2ndlevel caches, the front end, the out-of-order execution core, and the retire section). Instructionsand data are supplied to these units through the bus interface unit.

Processor Architecture

Dell inspiron 15 5555

The AMD64 architecture is a simple yet powerful 64-bit, backward-compatible extension of theindustry-standard (legacy) x86 architecture. It adds 64-bit addressing and expands register resourcesto support higher performance for recompiled 64-bit programs, while supporting legacy 16-bit and 32-bit applications and operating systems without modification or recompilation. It is the architecturalbasis on which new processors can provide seamless, high-performance support for both the vast bodyof existing software and 64-bit software required for higher-performance applications.

The need for a 64-bit x86 architecture is driven by applications that address large amounts of virtualand physical memory, such as high-performance servers, database management systems, and CADtools. These applications benefit from both 64-bit addresses and an increased number of registers. Thesmall number of registers available in the legacy x86 architecture limits performance in computation intensiveapplications. Increasing the number of registers provides a performance boost to many suchapplications.

Register set

Dell inspiron 3800

- Four 32-bit registers can be used as

∗ Four 32-bit register (EAX, EBX, ECX, EDX)

∗ Four 16-bit register (AX, BX, CX, DX)

∗ Eight 8-bit register (AH, AL, BH, BL, CH, CL, DH, DL)

- Some registers have special use

∗ECX for count in loop instructions.

- Two index registers

*16- or 32-bit registers

Used in string instructions

» Source (SI) and

destination (DI)

*Can be used as general                      purpose data registers

- Two pointer registers

*16- or 32-bit registers

* Used exclusively to

maintain the stack

- Control registers

*(E)IP

» Program counter

*(E) FLAGS

» Status flags

– Record status information about the result of the last

arithmetic/logical instruction

» Direction flag

– Forward/backward direction for data copy

» System flags

– IF : interrupt enable

– TF : Trap flag (useful in single-stepping)

Register set

Dell inspiron 15 5555

Instruction set

Dell inspiron 3800

- General Purpose Instruction

Data transfer

Binary integer arithmetic

Decimal arithmetic

Logic operations

Shift and rotate

Bit and byte operations

Program control

String

Flag control

Segment register operations

- X87 FPU Instruction

Floating-point

Integer

Binary-coded decimal (BCD) operands

- SIMD Instruction

MMX Instruction

SSE Instruction

- System Instruction

Instruction set

Dell inspiron 15  5555

The AMD64 architecture supports the full legacy x86 instruction set, with additional instructions tosupport long mode (see Table 1-1 on page 2 for a summary of operating modes). The application programminginstructions are organized into four subsets, as follows:

- General-Purpose Instructions—

These are the basic x86 integer instructions used in virtually allprograms. Most of these instructions load, store, or operate on data located in the  general-purposeregisters (GPRs) or memory. Some of the instructions alter sequential program flow by branchingto other program locations.

- Streaming SIMD Extensions Instructions (SSE)—

These instructions load, store, or operate on datalocated primarily in the YMM/XMM registers. 128-bit media instructions operate on the lowerhalf of the YMM registers. SSE instructions perform integer and floating-point operations onvector (packed) and scalar data types. Because the vector instructions can independently andsimultaneously perform a single operation on multiple sets of data, they are called single instruction,multiple-data (SIMD) instructions. They are useful for high-performance media andscientific applications that operate on blocks of data.

- Multimedia Extension Instructions—

These include the MMX™ technology and AMD 3DNow!™technology instructions. These instructions load, store, or operate on data located primarily in the

64-bit MMX registers which are mapped onto the 80-bit x87 floating-point registers. Like the SSEinstructions, they perform integer and floating-point operations on vector (packed) and scalar datatypes. These instructions are useful in media applications that do not require high precision.Multimedia Extension Instructions use saturating mathematical operations that do not generateoperation exceptions. AMD has de-emphasized the use of 3DNow! instructions, which have beensuperseded by their more efficient SSE counterparts. Relevant recommendations are provided in

Chapter 5, “64-Bit Media Programming” on page 237, and in the AMD64 Programmer’s ManualVolume 4: 64-Bit Media and x87 Floating-Point Instructions.

- x87 Floating-Point Instructions—

These are the floating-point instructions used in legacy x87

applications. They load, store, or operate on data located in the 80-bit x87 registers.

Some of these application-programming instructions bridge two or more of the above subsets.

Forexample,

there are instructions that move data between the general-purpose registers and the

YMM/XMM or MMX registers,

and many of the integer vector (packed) instructions can operate on

either YMM/XMM or MMX registers,

although not simultaneously.

If instructions bridge two or moresubsets,

their descriptions are repeated in all subsets to which they apply.

Memory Organization

The memory that the processor addresses on its bus is called physical memory . Physical

memory is organized as a sequence of 8-bit bytes. Each byte is assigned a unique address, calleda physical address . The physical address space ranges from zero to a maximum of 236 – 1(64 GBytes).

Virtually any operating system or executive designed to work with an IA-32 processor will usethe processor’s memory management facilities to access memory. These facilities provide

features such as segmentation and paging, which allow memory to be managed efficiently andreliably. Memory management is described in detail in Chapter 3, Protected-Mode MemoryManagement, in the Intel Architecture Software Developer’s Manual, Volume 3. The following segmented, or real-address mode.With the flat memory model , memory appears to a program as a single, continuousaddress space, called a linear address space . Code (a program’s instructions), data, andthe procedure stack are all contained in this address space. The linear address space is byteaddressable, with addresses running contiguously from 0 to 232-1. An address for any byte inthe linear address space is called a linear address .

With the segmented memory model, memory appears to a program as a group of independent

address spaces called segments . When using this model, code, data, and stacks are typically

contained in separate segments. To address a byte in a segment, a program must issue a logicaladdress , which consists of a segment selector and an offset.

The primary reason for using segmented memory is to increase the reliability of programs andsystems. For example, placing a program’s stack in a separate segment prevents the stack fromgrowing into the code or data space and overwriting instructions or data, respectively. Placingthe operating system’s or executive’s code, data, and stack in separate segments also protectsthem from the application program and vice versa.

Virtual Memory

Virtual memory consists of the entire address space available to programs. It is a large linear-addressspace that is translated by a combination of hardware and operating-system software to a smallerphysical-address space, parts of which are located in memory and parts on disk or other externalstorage media.

- 64-bit mode—

This mode uses a flat segmentation model of virtual memory. The 64-bit virtualmemoryspace is treated as a single, flat (unsegmented) address space. Program addresses accesslocations that can be anywhere in the linear 64-bit address space. The operating system can useseparate selectors for code, stack, and data segments for memory-protection purposes, but the baseaddress of all these segments is always 0. (For an exception to this general rule, see “FS and GS asBase of Address Calculation” on page 17.)

- Compatibility mode—

This mode uses a protected, multi-segment model of virtual memory, just as

in legacy protected mode. The 32-bit virtual-memory space is treated as a segmented set of addressspaces for code, stack, and data segments, each with its own base address and protection parameters. A segmented space is specified by adding a segment selector to an address.

Physical Memory

Physical memory is the installed memory (excluding cache memory) in a particular computer systemthat can be accessed through the processor’s bus interface. The maximum size of the physical memoryspace is determined by the number of address bits on the bus interface. In a virtual-memory system, thelarge virtual-address space (also called linear-address space) is translated to a smaller physicaladdressspace by a combination of segmentation and paging hardware and software.Paging is a mechanism for translating linear(virtual) addresses into fixed-size blocks called pages, which the operating system can move, as

needed, between memory and external storage media (typically disk). The AMD64 architecturesupports an expanded version of the legacy x86 paging mechanism, one that is able to translate the full64-bit virtual-address space into the physical-address space supported by the particularimplementation.

Memory Management

Memory management strategies translate addresses generated by programs into addresses in physical

memory using segmentation and/or paging. Memory management is not visible to application

programs. It is handled by the operating system and processor hardware. The following descriptiongives a very brief overview of these functions.

Modes of operation

Dell inspiron 3800

When writing code for an IA-32 processor, a programmer needs to know the operating mode theprocessor is going to be in when executing the code and the memory model being used. The relationshipbetween operating modes and memory models is as follows:

• Protected mode.

When in protected mode, the processor can use any of the memory

models described in this section. (The real-addressing mode memory model is ordinarily

used only when the processor is in the virtual-8086 mode.) The memory model used

depends on the design of the operating system or executive. When multitasking is implemented,individual tasks can use different memory models.

• Real-address mode.

When in real-address mode, the processor only supports the real address

mode memory model.

• System management mode.

When in SMM, the processor switches to a separate address

space, called the system management RAM (SMRAM). The memory model used to

address bytes in this address space is similar to the real-address mode model. (See Chapter

11, in the Intel Architecture Software Developer’s Manual, Volume 3, for more information

on the memory model used in SMM.)

Dell inspiron 15 5555

Modes of operation supported by the AMD64 architecture. In

most cases, the default address and operand sizes can be overridden with instruction prefixes. The

register extensions shown in the second-from-right column of Table 1-1 are those illustrated in

Long Mode

Long mode is an extension of legacy protected mode. Long mode consists of two submodes: 64-bitmode and compatibility mode. 64-bit mode supports all of the features and register extensions of the

AMD64 architecture. Compatibility mode supports binary compatibility with existing 16-bit and 32-bit applications. Long mode does not support legacy real mode or legacy virtual-8086 mode, and itdoes not support hardware task switching.

Throughout this document, references to long mode refer to both 64-bit mode and compatibility mode.

If a function is specific to either of these submodes, then the name of the specific submode is usedinstead of the name long mode.

64-Bit Mode

64-bit mode—a submode of long mode—supports the full range of 64-bit virtual-addressing andregister-extension features. This mode is enabled by the operating system on an individual codesegmentbasis. Because 64-bit mode supports a 64-bit virtual-address space, it requires a 64-bitoperating system and tool chain. Existing application binaries can run without recompilation incompatibility mode, under an operating system that runs in 64-bit mode, or the applications can also berecompiled to run in 64-bit mode.

Addressing features include a 64-bit instruction pointer (RIP) and an RIP-relative data-addressingmode. This mode accommodates modern operating systems by supporting only a flat address space,with single code, data, and stack space.

Register Extensions.

64-bit mode implements register extensions through a group of instructionprefixes, called REX prefixes. These extensions add eight GPRs (R8–R15), widen all GPRs to 64 bits,and add eight YMM/XMM registers (YMM/XMM8–15).

The REX instruction prefixes also provide a byte-register capability that makes the low byte of any ofthe sixteen GPRs available for byte operations. This results in a uniform set of byte, word,double word, and quad word registers that is better suited to compiler register-allocation.

64-Bit Addresses and Operands.

In 64-bit mode, the default virtual-address size is 64 bits

(implementations can have fewer). The default operand size for most instructions is 32 bits. For mostinstructions, these defaults can be overridden on an instruction-by-instruction basis using instructionprefixes. REX prefixes specify the 64-bit operand size and register extensions.

RIP-Relative Data Addressing. 64-bit mode supports data addressing relative to the 64-bit

instruction pointer (RIP). The legacy x86 architecture supports IP-relative addressing only in control

Opcodes.

A few instruction opcodes and prefix bytes are redefined to allow register extensions and

64-bit addressing. These differences are described in Appendix B “General-Purpose Instructions in

64-Bit Mode” and Appendix C “Differences Between Long Mode and Legacy Mode” in Volume 3.

Compatibility Mode

Compatibility mode—the second submode of long mode—allows 64-bit operating systems to runexisting 16-bit and 32-bit x86 applications. These legacy applications run in compatibility modewithout recompilation.Applications running in compatibility mode use 32-bit or 16-bit addressing and can access the first

4GB of virtual-address space. Legacy x86 instruction prefixes toggle between 16-bit and 32-bitaddress and operand sizes.

As with 64-bit mode, compatibility mode is enabled by the operating system on an individual codesegmentbasis. Unlike 64-bit mode, however, x86 segmentation functions the same as in the legacyx86 architecture, using 16-bit or 32-bit protected-mode semantics. From the application viewpoint,compatibility mode looks like the legacy x86 protected-mode environment. From the operatingsystemviewpoint, however, address translation, interrupt and exception handling, and system datastructures use the 64-bit long-mode mechanisms.

Legacy Mode

Legacy mode preserves binary compatibility not only with existing 16-bit and 32-bit applications butalso with existing 16-bit and 32-bit operating systems. Legacy mode consists of the following threesubmodes:

• Protected Mode—

Protected mode supports 16-bit and 32-bit programs with memorysegmentation, optional paging, and privilege-checking. Programs running in protected mode canaccess up to 4GB of memory space.

• Virtual-8086 Mode—

Virtual-8086 mode supports 16-bit real-mode programs running as tasksunder protected mode. It uses a simple form of memory segmentation, optional paging, and limited

protection-checking. Programs running in virtual-8086 mode can access up to 1MB of memoryspace.

• Real Mode—

Real mode supports 16-bit programs using simple register-based memory

segmentation. It does not support paging or protection-checking. Programs running in real modecan access up to 1MB of memory space.

Legacy mode is compatible with existing 32-bit processor implementations of the x86 architecture.

Processors that implement the AMD64 architecture boot in legacy real mode, just like processors thatimplement the legacy x86 architecture.

Throughout this document, references to legacy mode refer to all three submodes—protected mode,virtual-8086 mode, and real mode. If a function is specific to either of these submodes, then the nameof the specific submode is used instead of the name legacy mode

IO Mechanism

Dell inspiron 3800

Any program or task running on an IA-32 processor is given a set of resources for executing

instructions and for storing code, data, and state information. These resources (described brieflyin the following paragraphs and shown in Figure 3-1) make up the basic execution environmentfor an IA-32 processor. This basic execution environment is used jointly by the applicationprograms and the operating-system or executive running on the processor.

• Address Space. Any task or program running on an IA-32 processor can address a linear

address space of up to 4 GBytes (232 bytes) and a physical address space of up to 64

GBytes (236 bytes). (See Section 3.3.3., “Extended Physical Addressing” for moreinformation about addressing an address space greater than 4 GBytes.)

• Basic program execution registers . The eight general-purpose registers, the six segment

registers, the EFLAGS register, and the EIP (instruction pointer) register comprise a basic

execution environment in which to execute a set of general-purpose instructions. These

instructions perform basic integer arithmetic on byte, word, and doubleword integers,

handle program flow control, operate on bit and byte strings, and address memory.

• x87 FPU registers. The eight x87 FPU data registers, the x87 FPU control register, the

status register, the x87 FPU instruction pointer register, the x87 FPU operand (data) pointer

register, the x87 FPU tag register, and the x87 FPU opcode register provide an execution

environment for operating on single-precision, double-precision, and double extendedprecisionfloating-point values, word-, doubleword, and quadword integers, and binary

coded decimal (BCD) values.

• MMX™ registers. The eight MMX registers support execution of single-instruction,

multiple-data (SIMD) operations on 64-bit packed byte, word, and doubleword integers.

• XMM registers. The eight XMM data registers and the MXCSR register support

execution of SIMD operations on 128-bit packed single-precision and double-precision

floating-point values and on 128-bit packed byte, word, doubleword, and quadword

integers.

• Stack. To support procedure or subroutine calls and the passing of parameters between

procedures or subroutines, a stack and stack management resources are included in the

execution environment. The stack (not shown in Figure 3-1) is located in memory.

In addition to the resources provided in the basic execution environment, the IA-32 architectureprovides the following system resources. These resources are part of the IA-32 architecture’ssystem-level architecture. They provide extensive support for operating-system and systemdevelopmentsoftware. Except for the I/O ports, the system resources are described in detail inthe Intel Architecture Software Developer’s Manual, Volume 3: System Programming Guide.

• I/O Ports. The IA-32 architecture supports a transfers of data to and from input/output

(I/O) ports (see Chapter 12, Input/Output, in this volume).

• Control registers. The five control registers (CR0 through CR5) determine the operating

mode of the processor and the characteristics of the currently executing task (see the

section titled “Control Registers” in Chapter 3 of the Intel Architecture Software

Developer’s Manual, Volume 3)

IO Mechanism

Dell inspiron 15 5555

I/O devices allow the processor to communicate with the outside world, usually to a human or toanother system. In fact, a system without I/O has little utility. Typical I/O devices include a keyboard,mouse, LAN connection, printer, storage devices, and monitor. The speeds these devices must operateat vary greatly, and usually depend on whether the communication is to a human (slow) or to anothermachine (fast). There are exceptions. For example, humans can consume graphics data at very high

rates.There are two methods for communicating with I/O devices in AMD64 processor implementations.One method involves accessing I/O through ports located in I/O-address space (“I/O Addressing” onpage 94), and the other method involves accessing I/O devices located in the memory-address space

(“Memory Organization” on page 9). The address spaces are separate and independent of each other.I/O-address space was originally introduced as an optimized means for accessing I/O-device control

ports. Then, systems usually had few I/O devices, devices tended to be relatively low-speed, deviceaccesses needed to be strongly ordered to guarantee proper operation, and device protectionrequirements were minimal or non-existent. Memory-mapped I/O has largely supplanted I/O-address

space access as the preferred means for modern operating systems to interface with I/O devices.Memory-mapped I/O offers greater flexibility in protection, vastly more I/O ports, higher speeds, andstrong or weak ordering to suit the device requirements.

I/O Addressing

Access to I/O-address space is provided by the IN and OUT instructions, and the string variants ofthese instructions, INS and OUTS. The operation of these instructions are described in “Input/Output”

on page 68. Although not required, processor implementations generally transmit I/O-port addresses and I/O data over the same external signals used for memory addressing and memory data. Differentbus-cycles generated by the processor differentiate I/O-address space accesses from memory-addressspace accesses.

I/O-Address Space

Figure 3-18 on page 95 shows the 64 Kbyte I/O-address space. I/O ports can be addressed as bytes,words, or doublewords. As with memory addressing, word-I/O and doubleword-I/O ports are simply two or four consecutively-addressed byte-I/O ports. Word and doubleword I/O ports can be aligned onany byte boundary, but there is typically a performance penalty for unaligned accesses. Performance isoptimized by aligning word-I/O ports on word boundaries, and doubleword-I/O ports on doublewordboundaries.

Comparison

| Parameter | Dell Inspiron 3800 | Dell Inspiron 5555 |
| --- | --- | --- |
|  | Released in 2000, the Dell Inspiron 3800 was an affordable laptop that Dell branded as "Stylish and Affordable." Its starting price was $1,199. | Dell Inspiron 5555 is a Windows 10 Home laptop with a 15.60-inch display that has a resolution of 1366x768 pixels. It is powered by a APU Quad Core A10 processor and it comes with 8GB of RAM. |
| Processor : | Intel Pentium III @700 MHz, Intel Pentium III @600 MHz, Intel Celeron @600 MHz, or Intel Celeron @700 MHz | AMD E2-7110 APU, AMD A4-7210 APU @1.8 GHz, AMD A6-7310 APU @2 GHz, AMD A8-7410 APU @2.2 GHz, and AMD A10-8700P APU @1.8 GHz |
| Memory: | 32, 64, 96, 128, 192, 256, 384 or 512 MB of DDR RAM | 4 GB, 8 GB, 12 GB, or 16 GB DDR3L 1600 MHz RAM |
| Graphics: | ATI 2x AGP Rage Mobility with 8mb of video memory | AMD Radeon R2, R3, R4, R5, or R6 Integrated Graphics |
| Display: | 12.1" 800x600, or a 14.1" 1024x768 | Non-touch or touch 15.6" Dell True Life 1366x768 display |
| Storage: | 5gb, 6gb, 10gb, or 20gb Ultra ATA hard drive | Seagate Laptop Thin HDD SATA 500 GB, 1 TB, or 2 TB (5400 RPM) or 128 GB and 256 GB SSD |

Quotation